# Network Troubleshooting Lab Notebook

Project: Troubleshooting DHCP and Inter-VLAN Configured Enterprise Network

## Lab Objective

To implement and systematically troubleshoot DHCP and Inter-VLAN routing issues in an enterprise-style network using Cisco devices.

## Lab Environment

- Simulation Tool: Cisco Packet Tracer
- Devices: 1 Router, 1 Layer 2 Switch, Multiple PCs
- Routing Method: Router-on-a-Stick
- IP Assignment: DHCP

## Phase 1 – Physical & Basic Checks

Checked device power status, cabling, and interface states using:
- show ip interface brief
- show interfaces status

Observation: Interfaces must be up/up before proceeding.

## Phase 2 – VLAN Verification

Verified VLAN creation and port assignments using:
- show vlan brief

Observation: End devices must belong to correct VLANs.

## Phase 3 – Trunk Configuration Check

Validated trunking between switch and router using:
- show interfaces trunk

Observation: All required VLANs must be allowed on the trunk.

## Phase 4 – Inter-VLAN Routing Check

Verified router subinterfaces using:
- show ip interface brief
- show running-config

Observation: Each VLAN requires a correct subinterface with encapsulation dot1q.

## Phase 5 – DHCP Troubleshooting

Checked DHCP pool configuration and bindings using:
- show ip dhcp pool
- show ip dhcp binding

Observation: Incorrect network statements cause DHCP failure.

## Phase 6 – ACL & Traffic Flow Validation

Reviewed ACL behavior (if applied) using:
- show access-lists

Observation: ACLs can block DHCP or inter-VLAN traffic if misconfigured.

## Phase 7 – End-to-End Testing

Tested connectivity using:
- ping
- traceroute

Observation: Successful pings confirm routing and DHCP correctness.

## Common Faults & Fixes

- PC not receiving IP → Checked VLAN + DHCP pool
- No inter-VLAN ping → Checked trunk + router subinterfaces
- One VLAN working, another not → Check allowed VLANs

## Key Learnings

This lab strengthened understanding of enterprise network troubleshooting, DHCP behavior, VLAN segmentation, and structured OSI-based fault isolation.

## Conclusion

This project provided hands-on experience in troubleshooting DHCP and Inter-VLAN routing issues within an enterprise-style network. By following a structured, phase-wise and OSI-based approach, common faults related to VLAN configuration, trunking, router subinterfaces, and DHCP services were systematically identified and resolved.